Product Requirement Document (PRD)

| Project Code | VVDN_SPPS_2026 |
| --- | --- |
| Project Name | Smart Programmable Voltage Supply Module (85-265 V AC in , 5-20 V DC Out) |
| Revision | A1_001 |
| Date | 15th Oct 2026 |

Revision History

| Date | Revision No. | Description | Prepared/Revised By | Reviewed By | Approved By |
| --- | --- | --- | --- | --- | --- |
| 15th Oct 2026 | A1_001 | Smart Programmable Voltage Supply Module (85-265 V AC in , 5-20 V DC Out) | Sanskar Singh | Chitra Bhattacharya |  |

Customer Sign Off

| CUSTOMER SIGN-OFF FORM |  |  |
| --- | --- | --- |
| Customer | Technical In-Charge | Program In-Charge |
| Name: |  |  |
| Designation: |  |  |
| Email ID: |  |  |
| Address: |  |  |
| Comments: |  |  |
| Signature & Date: |  |  |
| VVDN | Technical In-Charge | Program In-Charge |
| Name: |  |  |
| Designation: | Project Owner Engineering | BU Head |
| Email ID: |  |  |
| Address: | VVDN Technologies B-22, Infocity Sector-34, Gurgaon-122001, Haryana, India Tel No: +91 124 4284250 Fax No: +91 124 4284251 | VVDN Technologies B-22, Infocity Sector-34, Gurgaon-122001, Haryana, India Tel No: +91 124 4284250 Fax No: +91 124 4284251 |
| Comments: |  |  |
| Signature & Date: |  |  |

Internal Sign Off

| INTERNAL SIGN-OFF |  |
| --- | --- |
| Level 1 | BU SME |
| Name: |  |
| Comments: |  |
| Signature & Date: |  |

Table of Contents

1	Introduction	7

1.1	Purpose	7

1.2	Document Conventions	7

1.3	Intended Audience and Reading Suggestions	7

1.4	Abbreviation	7

1.5	References	8

2	Product Overview	9

2.1	Product - Business Perspective	9

2.2	Product – Technical Perspective	9

2.3	Use Cases	10

2.4	Product Interfaces	10

2.4.1	Hardware Interfaces	11

2.4.2	Software Interfaces	12

3	Systems & Domains Involved	14

4	Product Requirement	15

4.1	System1 Requirements	15

4.1.1	Specific Requirements	15

4.2	System2 Requirements	16

4.2.1	Specific Requirements	16

5	Assumptions	16

6	Dependencies	17

7	Standard Deliverables & Release Order	18

7.1	Product Development Plan	18

7.2	System1 Development Plan	18

7.3	System2 Development Plan	19

8	Architecture 	20

8.1	System1: Domain1 Architecture	20

8.2	System1: Domain2 Architecture	20

8.3	System2: Domain1 Architecture	20

9	Bill Of Material 	21

9.1	System1: Product BOM	21

9.2	System2: Product BOM	21

10	Out of Scope	22

11	Open Items	22

List of Tables

Table 1: Abbreviation	8

Table 2: Reference	8

Table 3: Use Case	10

Table 4: Hardware Interfaces	12

Table 5: Software Interfaces	13

Table 6: Domain	14

Table 7: System1 Development Plan	19

Table 8:System1 Product BOM	21

Table 9:System2 Product BOM	22

List of Figures

Figure 1: Boundary Diagram	9

Figure 2: Major Use Case 1	10

Figure 3: Major Use Case 1	10

Figure 4: Flow Diagram	15

# Introduction

## Purpose

This document describes the system-level requirements for the design and development of 'VVDN_SPPS_2026' (Smart Programmable Voltage Supply Module) for internal VVDN and client test bench applications. The document will be used as a reference for further engineering documents, circuit schematics, PCB layout rules, and laboratory validation. The device's functional and technical requirements must meet all parameters specified herein. These requirements have been derived from client specifications and subsequent engineering design discussions.

This document captures all Electrical, Mechanical, and Hardware requirements to be delivered for the complete 4-Stage Power Supply PCBA module. VVDN_SPPS_2026 is the project ID for this project and it will be referenced in every document related to this project.

## Document Conventions

This PRD shall be taken as the sole authoritative reference baseline for all hardware requirements. Requirements marked with [MUST] are mandatory non-negotiable specifications essential for safety, electrical regulation, and compliance. Requirements marked with [SHOULD] represent performance targets where design trade-offs require SME approval. Requirements marked with [MAY] represent optional enhancements. All electrical units follow SI conventions.

## Intended Audience and Reading Suggestions

This PRD is made for the reference of Product Management, Hardware Architects, the Trainee Hardware Design Owner, Firmware Developers, Quality Assurance / DVT Engineers, and Production / DFM Engineers.

## Abbreviation

| Acronym | Definition |
| --- | --- |
| VVDN_SPPS_2026 | VVDN Smart Programmable Power Supply (Project Identifier) |
| PRD | Product Requirement Document |
| HDD | Hardware Design Document (Detailed Engineering Design Specification) |
| AC | Alternating Current (Grid Mains Input, 85 V – 265 V AC RMS) |
| DC | Direct Current (Regulated Output, 5.0 V – 20.0 V DC) |
| SELV | Safety Extra-Low Voltage (< 60 V DC limit per IEC 62368-1) |
| PFC | Power Factor Correction |
| CC | Constant Current Mode (Autonomous Hardware Current Limit) |
| CV | Constant Voltage Mode (Precision Closed-Loop Voltage Regulation) |
| DAC | Digital-to-Analog Converter (12-bit Voltage Setpoint Injection) |
| ADC | Analog-to-Digital Converter (Microcontroller Telemetry Sampling) |
| OVP | Over-Voltage Protection |
| OCP | Over-Current Protection |
| OTP | Over-Temperature Protection |
| SCP | Short-Circuit Protection |
| DMM | Digital Multimeter |
| CT | Current Transformer (AC Primary Current Sensing) |
| I2C | Inter-Integrated Circuit Serial Communication Bus |
| SPI | Serial Peripheral Interface Bus |
| UART | Universal Asynchronous Receiver-Transmitter |
| PWM | Pulse Width Modulation |
| EMI | Electromagnetic Interference |
| EMC | Electromagnetic Compatibility |
| NTC | Negative Temperature Coefficient Thermistor (Inrush Limiter) |
| MOV | Metal Oxide Varistor (Surge & Transient Voltage Clamp) |
| ESR | Equivalent Series Resistance |
| DVT | Design Verification Testing |
| ATE | Automated Test Equipment |
| DUT | Device Under Test |

Table 1: Abbreviation

## References

| SL No | Description (with Version & Reference) |
| --- | --- |
| REF-001 | Statement of Work (SOW): Smart Programmable Voltage Supply Module v1.0, VVDN Technologies |
| REF-002 | Texas Instruments LM5176: High-Performance 4-Switch Synchronous Buck-Boost Controller Datasheet (SNVSAR4) |
| REF-003 | Texas Instruments UCC28740: Constant-Voltage Constant-Current Isolated Flyback Controller Datasheet (SLUSBL2) |
| REF-004 | VVDN Hardware Design Document (HDD): Smart Programmable Power Supply Circuit Design, Derivations & Schematics |

Table 2: Reference

# Product Overview

## Product - Business Perspective

The Smart Programmable Voltage Supply Module is an intelligent, high-density benchtop and automated test equipment (ATE) power supply delivering a tightly regulated, programmable DC voltage output from 5.0 V to 20.0 V at currents up to 3.0 A (60 W continuous output power) from universal grid AC mains input (85 V to 265 V AC RMS, 47 Hz to 63 Hz).

Traditional bench power supplies rely on bulky 50 Hz iron-core transformers, inefficient linear regulators, and manual dials, which lead to high thermal dissipation, large footprint, and lack of automated telemetry. This product eliminates those limitations by integrating a universal mains (QR) Flyback converter, an ultra-efficient 4-switch synchronous Buck-Boost post-regulator, real-time dual-domain power telemetry, and autonomous analog hardware protection into a single compact board.

All requirements and functional parameters in this document are captured from the perspective of high-efficiency benchtop prototyping and automated modular test racks.

## Product – Technical Perspective

The Smart Programmable Power Supply is a 4-stage AC-to-DC conversion and regulation system in which the power stage and mixed-signal microcontroller work together to deliver precise, clean, and safe DC power. The module connects directly to universal grid AC mains (85V–265V AC) through an input protection and EMI filter stage, rectifies it to a high-voltage bulk DC rail (+120V to +375V DC), isolates and steps it down via a Flyback stage to an intermediate +24.0V DC bus (72W), and regulates it through a synchronous 4-switch Buck-Boost stage to provide the commanded 5.0V to 20.0V DC output at up to 3.0A.

The onboard microcontroller (STM32G474) acts as the system brain by reading dual-domain electrical telemetry (primary AC input power and secondary DC load power), computing real-time conversion efficiency, updating the 1.3-inch OLED dashboard, and driving the precision 12-bit DAC voltage steering network. Circuit protection relies on robust, standard, industry-proven hardware protections: primary AC fuse, inrush NTC, surge MOV, primary cycle-by-cycle Flyback current limiting, and secondary LM5176 built-in cycle-by-cycle overcurrent protection with thermal shutdown.

In simple terms, the power conversion stages handle the heavy electrical energy flow across certified 3000V RMS galvanic isolation, while the built-in controller protections and MCU provide safety, precision regulation, and live telemetry.

The primary power supply functionality includes:
• Universal AC Grid Operation: Accepts 85V to 265V AC RMS (47–63 Hz) without manual line selector switches.
• Reinforced Galvanic Isolation: Certified 3000V AC RMS (1 min) isolation meeting IEC 62368-1 SELV.
• Intermediate Regulated Rail: Flyback converter generates a stable +24.0V DC bus (72W) with > 90% peak efficiency.
• Wide Programmable DC Output: 4-switch synchronous Buck-Boost post-regulator delivers 5.0V to 20.0V DC at up to 3.0A continuous (60W continuous power).
• High-Resolution DAC Voltage Steering: 12-bit DAC network achieves 10mV monotonic setpoint resolution.
• Built-in Cycle-by-Cycle Hardware Current Limiting: LM5176 monitors output current through a dedicated 10mΩ shunt across its CSP/CSN pins, natively limiting peak inductor current on every switching cycle without external analog complexity.
• Built-in Controller Protections: Integrated input fuse (2A), NTC inrush limiter (10Ω), MOV transient clamp (300V), LM5176 internal thermal shutdown (165°C), and Under-Voltage Lockout (UVLO).
• Real-Time Dual-Domain Telemetry: Measures AC input power and DC load power, computing live efficiency (η) at 10 Hz.
• Interactive Local Dashboard: 1.3" OLED display shows live V, I, P, and efficiency meters; dual rotary encoders allow intuitive tuning; front-panel LEDs indicate CV, Current Limit, and Fault states.
• Low Standby Power: Consumes < 150 mW in idle state when output is disabled.

Figure 1: Boundary Diagram / Simplified 4-Stage Power Flow Block Diagram

Table 3: High Level Technical Requirement

| S.No | Parameters | Inputs For SPPS Board Design |
| --- | --- | --- |
| 1 | Input Voltage Range | 85 V to 265 V AC RMS, 47 Hz to 63 Hz (Universal Grid Compatibility) |
| 2 | Continuous Output Current | 0.0 A to 3.0 A DC Continuous across full voltage range |
| 3 | Programmable Output Voltage | 5.0 V to 20.0 V DC |
| 4 | Maximum Output Power | 60.0 W Continuous DC Output (20.0 V @ 3.0 A) |
| 5 | Intermediate Bus Voltage | +24.0 V DC ± 1.0% (72.0 W continuous rating from primary Flyback stage) |
| 6 | Output Voltage Ripple | < 25 mV pk-pk under full 3.0 A load (measured across 20 MHz bandwidth) |
| 7 | End-to-End System Efficiency | > 85.0% @ 230 V AC Full Load (Flyback > 90%, Buck-Boost > 95%) |
| 8 | Galvanic Safety Isolation | 3,000 V AC RMS (1 minute withstand) meeting IEC 62368-1 SELV |
| 9 | Overcurrent Protection | LM5176 built-in cycle-by-cycle hardware peak current limit via CSP/CSN |
| 10 | User Interface & Control | 1.3" I2C OLED display, dual rotary encoders, Status LEDs |
| 11 | Conducted & Radiated EMC | Compliant with CISPR 32 / EN 55032 Class B limits |

The product is designed as a unified Embedded Hardware Power Supply Module (System 1). Primary hardware stages include AC Mains Protection & EMI Filter, GBU606 Bridge Rectifier & Bulk Capacitor (+120V to +375V DC), Stage 1 Isolated Flyback Converter (+24V DC, 72W), Stage 2 Synchronous 4-Switch Buck-Boost Post-Regulator (5V–20V / 3A DC), and Analog DAC Steering & Built-in Hardware Protections.

## Use Cases

The primary power supply functionality includes four operating modes:
• Constant Voltage (CV) Mode: Regulates output voltage within ±0.2% across varying load currents below the current limit.
• Current-Limited (OCP) Mode: Restricts peak inductor current cycle-by-cycle via LM5176 CS/CSG sense during overload or short-circuit.
• MCU Supervisory Protection Mode: Safely disables switching via LM5176 ENABLE pin on persistent overload or over-temperature.
• Standby / Idle Mode: Output power stage disabled while +24V intermediate rail is maintained with standby power < 150mW.

The benchtop operator interacts with the power supply using front-panel dual rotary controls and a 1.3" graphical OLED display. Power is delivered to the device under test (DUT) via heavy-duty 4mm binding posts, and operational status is indicated via front-panel mode LEDs.

| S.N. | Usecase |
| --- | --- |
| 1 | Universal AC mains soft startup and inrush current suppression (< 25 A peak) |
| 2 | AC full-wave rectification and high-voltage DC bulk energy holdup (> 10 ms) |
| 3 | Galvanically isolated Flyback conversion (+24.0 V DC, 72 W) |
| 4 | Synchronous 4-switch Buck-Boost step-down/step-up post-regulation (5 V – 20 V DC) |
| 5 | High-precision 10 mV digital voltage steering via 12-bit DAC summing network |
| 6 | LM5176 built-in cycle-by-cycle overcurrent protection via CSP/CSN pins |
| 7 | Primary Flyback cycle-by-cycle current limit via UCC28740 CS resistor |
| 8 | Controller internal thermal shutdown and board temperature monitoring |
| 9 | Primary AC input power measurement via isolated delta-sigma amplifier and CT |
| 10 | Secondary DC load voltage, current, and power monitoring via 16-bit I2C monitor |
| 11 | Real-time conversion efficiency computation and 1.3" OLED dashboard display update |
| 12 | Local front-panel rotary encoder tuning, tactile output enable, and status LED indication |

Table 4: Product Usecase

## Product Interfaces

Physical hardware interfaces include an IEC 320-C14 AC inlet with integrated fuse drawer, heavy-duty 4mm gold binding posts, dual optical rotary encoders for voltage and current limit tuning, a tactile output enable push-button with status LED, a 1.3" monochrome OLED display, and dedicated PCB bring-up test points.

### Hardware Interfaces

The hardware interfaces provide rugged, low-impedance connections: binding posts carry 3.0A continuous load current with < 5mΩ contact resistance; IEC inlet provides 250V / 10A rating with earth grounding; test points allow high-voltage differential probing across all switching nodes.

| Sl. No | Entity 1 | Entity 2 | Interface | Usage Description |
| --- | --- | --- | --- | --- |
| 1 | AC Grid Mains | SPPS Board | IEC 320-C14 Inlet | Universal 85V–265V AC RMS, 50/60Hz grounded power input with 2A fuse |
| 2 | SPPS Board | DUT / Load | 4mm Binding Posts | Regulated 5.0V–20.0V DC / 3.0A power delivery to device under test |
| 4 | SPPS Board | Local Operator | 1.3" OLED Display | Real-time graphical dashboard for V, I, P, efficiency (η), and operating mode |
| 5 | SPPS Board | Local Operator | Output Enable Button | Tactile push-button for safe electronic enable/disable of output power |
| 6 | SPPS Board | Local Operator | Status Mode LEDs | Visual indication of Output Active (Green), Limit (Amber), and Fault (Red) states |
| 7 | SPPS Board | Debug Programmer | SWD Header | Microcontroller firmware flashing and debug tracing |
| 8 | SPPS Board | Lab Oscilloscope | PCB Test Points | Diagnostic test loops for measuring intermediate rail, ripple, and noise |
| 9 | Primary Side | Secondary Side | 6.4mm Milled Slot | Physical 2.0mm air slot beneath transformer enforcing IEC 62368-1 isolation |

Table 5: Hardware Interface

### Software Interfaces

The SPPS module contains an onboard STM32G474 mixed-signal microcontroller that coordinates telemetry sampling, closed-loop DAC voltage steering, status display, and fault handling. The following table describes the interface, protocol/mode, and usage of each interface:

| S. No | Entity 1 | Entity 2 | Interface | Interface Protocol / Mode | Usage Description |
| --- | --- | --- | --- | --- | --- |
| 1 | STM32G474 MCU | INA226 Monitor | Secondary Telemetry | I2C | Reads 16-bit DC output voltage, current, and bus power at 10 Hz rate |
| 2 | STM32G474 MCU | 1.3" OLED (SSD1306) | Display Dashboard | I2C | Updates numerical readouts, mode text, and efficiency bar graph at 10 Hz |
| 3 | STM32G474 MCU | AMC1311 Iso-Amp | Primary AC Voltage | Analog ADC Channel | Samples galvanically isolated AC mains voltage waveform for Pac calculation |
| 4 | STM32G474 MCU | Current Transformer | Primary AC Current | Analog ADC Channel | Samples burden resistor voltage from 1000:1 CT for AC mains current sensing |
| 5 | STM32G474 MCU | LM5176 FB Node | Voltage Steering | Internal 12-Bit DAC | Sends current into summing node to set Vout between 5.0V and 20.0V |
| 6 | STM32G474 MCU | Rotary Encoders | User Input | Timer Encoder Interface | Rotary dials with debounced push-switches |
| 7 | STM32G474 MCU | Status LEDs | Status Annunciation | GPIO Digital Outputs | Drives Green (Active), Amber (Limit), and Red (Fault) status indicator LEDs |
| 8 | STM32G474 MCU | LM5176 Enable Pin | Output Control | GPIO Digital Output | Controls post-regulator run state during standby, soft-start, and fault disable |
| 9 | STM32G474 MCU | Host PC / ATE | Calibration & SCPI | UART | Optional serial channel for automated calibration and factory test commands |

Table 6: Software Interfaces

# Systems & Domains Involved

The engineering domains involved in the development of the SPPS module are summarized in Table 7 below:

| Domain Names | System 1 (SPPS Device) | Trainee Responsibility Scope | Mentor Review Mechanism |
| --- | --- | --- | --- |
| Hardware - HW | Yes | 4-Stage power conversion schematics, component selection, layout, and bring-up | Gate Review |
| Mechanical - ME | Yes | Enclosure integration, thermal copper heatsinking, binding post cutouts | Mechanical Review |
| Firmware - FW | Yes | Bare-metal drivers (ADC, DAC, I2C, Timers), OLED dashboard, efficiency math | Firmware Review |
| Test Firmware | Yes | Dedicated bring-up routines for peripheral check, DAC sweep, and zeroing | Lab Bring-up Sign-off |

Table 7: Domain

# Product Requirement

## System1 Requirements

System 1 represents the physical Hardware Power Supply Module, comprising the universal AC input, EMI filter, isolated Flyback stage, 4-switch synchronous Buck-Boost post-regulator, built-in controller hardware protection, and microcontroller sensing. The detailed engineering specifications are defined in the tables below:

Table 4.1: AC-DC Isolated Flyback Power Stage Engineering Requirements

| Req ID | Parameter / Feature | Specification / Target |
| --- | --- | --- |
| FR-01 | AC Input Voltage Range | 85 V – 265 V AC RMS [MUST] |
| FR-02 | Intermediate DC Bus Voltage | +24.0 V DC ± 1.0% [MUST] |
| FR-03 | Continuous Primary Power | 72.0 W Continuous [MUST] |
| FR-04 | Peak Power Capability | 84.0 W for 500 ms [SHOULD] |
| FR-05 | Primary Stage Efficiency | > 90.0% @ 230V AC Full Load [MUST] |
| FR-06 | Safety Galvanic Isolation | 3,000 V AC RMS (1 min) [MUST] |
| FR-07 | Cold-Start Inrush Current | < 25.0 A Peak [MUST] |
| FR-08 | No-Load Standby Power | < 150 mW Standby [SHOULD] |

Table 4.2: 4-Switch Synchronous Buck-Boost Post-Regulator Requirements

| Req ID | Parameter / Feature | Specification / Target | Condition |
| --- | --- | --- | --- |
| BB-01 | Output Voltage Range | 5.0 V – 20.0 V DC Programmable [MUST] | Continuous linear adjustment |
| BB-02 | Continuous Output Current | 0.0 A – 3.0 A DC Continuous [MUST] | Across full 5V–20V voltage span |
| BB-03 | Maximum Output Power | 60.0 W Continuous Output [MUST] | Vout = 20.0V, Iout = 3.0A |
| BB-04 | Output Voltage Ripple | < 25 mV pk-pk ripple [MUST] | Full load 3A, 20MHz bandwidth |
| BB-05 | Buck-Boost Efficiency | > 95.0% @ 20V/3A; > 92.0% @ 5V/3A [MUST] | Vin = 24V, Sync Rectification |
| BB-06 | Switching Frequency | 300 kHz ± 10% [MUST] | Fixed frequency PWM |
| BB-07 | Hardware Current Limiting | Cycle-by-cycle current limit via CSP/CSN [MUST] | Overload condition |

Table 4.3: Real-Time Dual-Domain Telemetry & Monitoring Requirements

| Req ID | Parameter / Feature | Specification / Target | Condition |
| --- | --- | --- | --- |
| TM-01 | Primary AC Voltage Sensing | Isolated AMC1311 (0-300V AC) [MUST] | ± 1.0% accuracy across 85-265V |
| TM-02 | Primary AC Current Sensing | Current Transformer 1000:1 [MUST] | ± 1.5% accuracy from 0.1A–2A |
| TM-03 | Dual-Domain Telemetry Rate | 10.0 Hz Real-Time Loop Rate [MUST] | Pac, Pdc, and η computed at 100ms |
| TM-04 | Galvanic Isolation Barrier | 3,000 V AC RMS (1 min) [MUST] | Primary-to-secondary isolation |
| TM-05 | Calibration Test Points | Dedicated PCB Test Loops [MUST] | Vout, bulk DC, and aux rails |

Table 4.4: Protection, Safety, EMC & Environmental Requirements

| Req ID | Parameter / Feature | Specification / Target | Condition |
| --- | --- | --- | --- |
| PR-01 | Input Fuse Protection | 2.0A Ceramic Fuse [MUST] | AC mains short-circuit |
| PR-02 | Input Inrush Current Limit | 10Ω NTC Thermistor (< 25A peak) [MUST] | Cold start at 265V AC |
| PR-03 | Built-in Controller OCP | LM5176 cycle-by-cycle CSP/CSN clamp [MUST] | Load current > 3.5A limit |
| PR-04 | Over-Temperature (OTP) | LM5176 internal shutdown (165°C) [MUST] | Severe thermal rise |
| PR-05 | Safety Compliance Standard | IEC 62368-1 Reinforced Isolation [MUST] | SELV touch-safe (< 60V DC) |
| PR-06 | Creepage & Clearance | Creepage ≥ 6.4mm; Clearance ≥ 5.0mm [MUST] | Primary-Secondary milled slot |
| PR-07 | Conducted & Radiated EMC | CISPR 32 / EN 55032 Class B [MUST] | 150 kHz – 1 GHz emission limits |
| PR-08 | Operating Ambient Temp | –20°C to +50°C Full Load [MUST] | Natural convection in chassis |

### Specific Requirements

Specific hardware layout constraints mandate a physical 6.4 mm surface creepage distance and a 2.0 mm milled air slot through the PCB substrate beneath the transformer and optocoupler barriers to guarantee IEC 62368-1 reinforced safety isolation.

#### Inside Bridge Rectifier & High-Voltage Bulk DC Subsystem :

| BRIDGE RECTIFIER & BULK FILTER SEQUENCE |  |  |
| --- | --- | --- |
| Step 1 | Filtered AC Input | Takes 85V–265V AC RMS sinusoidal voltage from Stage 1 EMI filter. |
| Step 2 | Full-Wave Rectification | GBU606 single-phase bridge rectifies AC into 100Hz/120Hz unipolar DC pulses (120Vpk–375Vpk). |
| Step 3 | Bulk Capacitive Filtering | 100µF/450V electrolytic cap smooths ripple and guarantees > 10ms holdup time across line dropouts. |
| Step 4 | High-Frequency Decoupling | 100nF/630V ceramic cap bypasses fast 80kHz primary switching edge ringing. |

## System2 Requirements

System 2 represents the Secondary Stage 4-Switch Synchronous Buck-Boost Post-Regulator, Analog Closed-Loop Control, DAC Steering Network, and Autonomous Hardware CC Protection Subsystem. The detailed electrical hardware specifications are defined in Table 4.5 below:

Table 4.5: Analog Closed-Loop Control, DAC Steering & Hardware Protection Specifications

| Req ID | Parameter / Feature | Specification / Target |
| --- | --- | --- |
| CT-01 | DAC Voltage Steering Network | Resistor summing into LM5176 FB node [MUST] |
| CT-02 | Current Sense Resistor | 10 mΩ Metal Element Shunt, 3W, 0.1% [MUST] |
| CT-03 | MCU Supervisory Shutdown | GPIO pulling LM5176 ENABLE pin low [MUST] |
| CT-04 | Auxiliary Bias Regulation | +14V Flyback VCC, +12V gate, +5V, +3.3V rails [MUST] |

### Specific Requirements

#### Inside Stage 3: Converter DC-DC (Isolated Flyback) Subsystem

Stage 3 provides reinforced galvanic isolation (3000V RMS) and steps down high-voltage bulk DC power to an intermediate +24.0V DC bus (72W). The controller uses an industry-standard UC3843 running at a constant uniform clock.

| STAGE 3: UNIFORM-FREQUENCY FLYBACK DC-DC CONVERTER SEQUENCE |  |  |
| --- | --- | --- |
| Step 1 | High-Voltage DC Input | Receives +120V to +375V DC bulk bus into primary switching cell. |
| Step 2 | Uniform-Frequency PWM | UC3843 controller operates at constant uniform clock. |
| Step 3 | 3000V RMS Safety Barrier | PQ26/20 transformer (Lp=220µH) with Furukawa TEX-E wire provides reinforced galvanic isolation. |
| Step 4 | Schottky Barrier Rectification | MBR10100 100V/10A Schottky diode provides simple, robust passive rectification with high reliability. |
| Step 5 | +24.0V DC Intermediate Rail | Low-ESR LC filter delivers stable +24.0V DC (72W). |

# Assumptions

The following points are considered as assumptions for further project execution:
• Grid AC mains is nominal sinusoidal waveform with Total Harmonic Distortion (THD) ≤ 5%.
• Operating ambient temperature is -20°C to +50°C under natural convection airflow.
• Secondary DC ground is floating with reinforced galvanic isolation (≥ 3000V RMS) from Earth and AC mains.
• Connected prototype loads are predominantly resistive or capacitive (C_load ≤ 1000 µF).

# Dependencies

The following dependencies are included in the design and development of the SPPS module:
• Component sourcing and lead times for TI LM5176, UCC28740, INA226, AMC1311, and ST STM32G474.
• Laboratory equipment availability: AC source, DC electronic load, 3000V RMS Hi-Pot tester, and thermal camera.
• Mechanical front-panel drawings for binding post spacing (19.05mm)
• Firmware device drivers for SSD1306 OLED display, INA226 power monitor, and STM32 internal 12-bit DAC.

# Standard Deliverables & Release Order

The hardware deliverables and release documentation across project milestones are defined in the table below:

## Product Development Plan

Phase 1 — Requirement & Architecture and Class A BOM:
Definition of functional and electrical requirements; 4-stage system architecture design; block diagram and interface specifications.

Phase 2 — Component Selection & Circuit Design:
Transformer PQ26/20 magnetics sizing; component stress derating calculations; closed-loop feedback and DAC steering design; companion HDD authoring.

Phase 3 — Schematic Design and Review:
Complete 4-stage Altium schematics capture.

Phase 4 — PCB Layout & DFM Review:
PCB stackup; switching loop routing; DFM/DFA checks.

Phase 5 — Prototype Bring-up & Design Validation Plan (DVT):
PCB fabrication and assembly (5 units); cold resistance checks; supervised Variac bring-up; line/load regulation, ripple, efficiency, and Hi-Pot validation.

## System1 Development Plan

| System Name | Domain Name | Deliverables Domain3 [Embedded Software] Domain4 |  |  |
| --- | --- | --- | --- | --- |
| VVDN_SPPS_2026 | Systems / PRD | Product Requirement Document (PRD) — Baseline Specifications & Acceptance Criteria |  |  |
| SPPS Module | Hardware (HW) | Hardware Design Document (HDD) with complete circuit derivations & calculations |  |  |
| SPPS Module | Hardware (HW) | Altium Designer Complete Schematics |  |  |
| SPPS Module | Hardware (HW) | Class A Engineering Bill of Materials (BOM) with manufacturer part numbers |  |  |
| SPPS Module | Hardware (HW) | 4-Layer PCB Layout & Gerber Fabrication Package |  |  |
| SPPS Module | Hardware (HW) | Design Verification Testing (DVT) Report & Acceptance Certificate |  |  |
| SPPS Module | Firmware (FW) | Microcontroller Telemetry & Display Firmware Source Code |  |  |

Table 8: Standard Deliverables & Release Order

# Architecture

The power supply hardware architecture consists of two galvanically isolated power conversion stages followed by synchronous post-regulation and built-in controller hardware protection:

## Inside Stage 4 (Synchronous 4-Switch Buck-Boost Post-Regulator Subsystem)

| STAGE 4: 4-SWITCH BUCK-BOOST POST-REGULATOR SEQUENCE |  |  |
| --- | --- | --- |
| Step 1 | +24.0V DC Rail Input | Accepts +24.0V DC intermediate rail from Stage 3 Flyback. |
| Step 2 | 4-Switch Synchronous Bridge | TI LM5176 modulates four 40V MOSFETs in H-bridge around a 10µH flat-wire power inductor. |
| Step 3 | Seamless Mode Transitions | Operates in pure Buck mode for 5V–18V outputs (>96% eff) and 4-switch Buck-Boost mode near 20V (>95% eff). |
| Step 4 | Direct PWM Duty Cycle Voltage Control | MCU modulates 300 kHz PWM duty cycle (D = 20.8% for 5V to 83.3% for 20V) to directly command output voltage. |
| Step 5 | Cycle-by-Cycle Current Limit | LM5176 senses current via 10mΩ shunt across CSP/CSN, natively clamping peak inductor current on every 300 kHz cycle. |

Figure 6: Inside Stage 4 - 4-Switch Buck-Boost Post-Regulator Subsystem Architecture

## System Microcontroller: MCU System Interaction, State Machine & Monitoring Architecture

Figure 11: MCU System Interaction, State Machine & Monitoring Architecture

## System2: Domain1 Architecture

< Architecture Diagram with caption, shall be inserted here. No explanation of architecture is required. Below architecture diagram, there shall be a note “This is the proposed architecture and it might get changed during the design stage” and “Ensure the license compliance for usage of open source tools”.>

# Bill Of Material

Note: This is the expected BOM and it might get changed during the design stage. The BOM does not contain any pricing/cost information.

## System1: Product BOM

Refer to detailed bill of materials in the respective subsystem tables below.

| Sl.No | Description | Qty | Mfg | Mfg Part No |
| --- | --- | --- | --- | --- |
| Subsystem 1: AC Mains Protection, EMI Filtering & Rectifier Stage (Electrical) |  |  |  |  |

Table 9.1: System 1 Product BOM (Primary & Flyback Stages)

## System2: Product BOM

Refer to detailed bill of materials in the respective subsystem tables below.

| Sl.No | Description | Qty | Mfg | Mfg Part No |
| --- | --- | --- | --- | --- |
| Subsystem 1: 4-Switch Synchronous Buck-Boost Post-Regulator (Electrical) |  |  |  |  |

Table 9.2: System 2 Product BOM (Buck-Boost, Protection & Control)

# Out of Scope

The following points are not included in the scope of project:

- Active Power Factor Correction (Active Boost PFC) — passive EMI filtering is used since < 75W is exempt under IEC 61000-3-2.

- Complex External Protection Circuits — dedicated analog crowbar thyristors and discrete fast comparator pull-down networks are excluded in favor of built-in controller protection and MCU supervisory shutdown.

- Three-phase 400V/480V AC industrial grid input operation (module is single-phase 85V–265V AC RMS only). Negative/bipolar rails and wireless IoT interfaces are excluded.

# Open Items

The following points are included in the scope of project but not included in urrent revision:

- Current shunt value trade-off: Evaluating 5mΩ vs 10mΩ for thermal dissipation vs low-current sensing resolution.

- Auxiliary 24V DC barrel input option: Evaluating a 24V DC auxiliary input to bypass AC mains for vehicle/field use.

- Secondary heatsink copper area optimization: Evaluating 2oz vs 3oz copper pours for D2PAK primary FET.