Stage 3: Converter DC-DC (Isolated Flyback) Subsystem

Internal Subsystem Architecture, Component Selection & Engineering Requirements Table

| Functional Sub-Block | Circuit Ref | Selected Component / IC | Key Specifications & Ratings | Functional Role & Performance Target |
| --- | --- | --- | --- | --- |
| HVDC Input Interface | +HVDC | Stage 2 Bulk DC Bus Link | +120 V to +375 V DC operating span | Low-impedance high-voltage input rail feeding transformer primary winding and snubber network. |
| Quasi-Resonant Controller | U1 | Texas Instruments UCC28740DR (SOIC-7) | Valley-switching detector, aux ZCD, optical FB pin, cycle-by-cycle CS | Modulates switching frequency (up to 80 kHz) to turn on primary MOSFET at drain voltage resonance minimums, slashing turn-on capacitive losses by > 60%. |
| Primary Power Switch | Q1 | Infineon IPB80R290P7 (CoolMOS P7, D2PAK) | 800 V, 17 A, R_DS(on) = 0.29 Ω, V_DS,peak = 599 V | Low-loss primary switching element. Derated to 74.9% of breakdown voltage rating (strictly < 75% limit). Conduction loss <= 0.57 W. |
| Primary Current Sensing | R_CS | Precision Sense Resistor Network | Low-inductance metal foil, 0.25 Ω, 1%, 1 W | Senses primary switch peak current (I_pk = 2.96 A) on UCC28740 CS pin for cycle-by-cycle overcurrent protection (OCP). |
| RCD Snubber Clamp | D_snub R_snub C_snub | Diodes Inc. US1M + 3x 68 kΩ in parallel (22.6 kΩ, 3W) + 1.0 nF / 1 kV Ceramic | Diode: 1000 V, 1 A, t_rr = 75 ns; Cap: 1 nF / 1000 V ceramic; Clamp: ~216 V clamp level | Absorbs transformer primary leakage inductance energy (L_lk approx 2.2 uH); clamps peak drain-source voltage spike safely below 650 V peak. |
| Flyback Power Transformer | TX1 | Ferroxcube PQ26/20 (3C95 Ferrite Core) / Custom | L_p = 220 uH +/- 10%, B_max = 0.148 T (B_sat = 0.38 T), N_p = 36 T | Primary energy storage inductor (72 W throughput, I_pk = 2.96 A, low core loss, air gap l_g = 0.52 mm). |
| Auxiliary Bias & Demag Winding | TX1_AUX | N_aux = 4 T (0.25 mm wire) + Ultra-fast diode & 10 uF MLCC | Output: +14.0 V DC bias rail | Self-powers UCC28740 VCC pin after initial high-voltage startup and provides Zero Crossing Detection (ZCD) for core demagnetization timing. |
| Galvanic Safety Isolation Barrier | TX1_ISO | Furukawa TEX-E Triple-Insulated Wire (TIW) | N_s = 6 T (N_p:N_s = 6:1), Reinforced insulation, >= 6.4 mm creepage | Conforms to IEC 62368-1 safety standards. Provides certified >= 3000 V RMS dielectric withstand between primary hot and secondary touch-safe SELV domains. |
| Smart Synchronous Rectifier Driver | U2 | Monolithic Power Systems MP6908GJ (TSOT23-6) | Fast turn-off (35 ns gate fall), V_DS drain-sense detection | Autonomous gate driver for secondary sync switch; eliminates external sync timing coils and prevents shoot-through during DCM/QR resonant ringing. |
| Synchronous Rectifier Switch | Q_SR | Infineon BSC052N06NS (OptiMOS 5, SuperSO8) | 60 V, R_DS(on) = 5.2 mΩ, I_RMS,sec = 7.61 A | Replaces lossy Schottky diode (V_F = 0.6 V); cuts secondary rectification dissipation from 1.80 W to 0.42 W (saving 1.38 W continuous heat, heatsink-free). |
| Secondary LC Output Filter | C_sec L_sec | Panasonic OS-CON 470 uF / 35 V Polymer + 2.2 uH Inductor + 22 uF MLCC | Ultra-low ESR (< 15 mΩ), Delta V_ripple < 50 mV pk-pk | Suppresses high-frequency 80 kHz switching pulses and secondary flyback current spikes to form clean intermediate DC rail. |
| Isolated Closed-Loop Feedback | OPTO1 U3 | Everlight EL817(S)(TA) Optocoupler + TI TL431AIDBZR Shunt Regulator | 5000 V RMS optical isolation, 2.495 V internal bandgap, 0.5% | Secondary-side precision voltage divider senses the +24.0 V rail and transfers feedback error signal across isolation barrier to UCC28740 FB node. |
| Intermediate Rail Output | +24V_BUS | Intermediate DC Power Distribution Bus | +24.0 V DC +/- 1.0%, 72.0 W continuous (3.0 A), 84 W peak | Delivers stable intermediate regulated power to Stage 4 (Synchronous Buck-Boost Post-Regulator) and secondary auxiliary housekeeping LDOs. |

Note: This table is pre-formatted with standard cell margins, repeating headers, and fixed column widths. You can select the entire table in Word, copy it (Ctrl+C), and paste it (Ctrl+V) directly into your PRD document.