Stage 3: Converter DC-DC (Uniform-Frequency Isolated Flyback) Subsystem

Internal Subsystem Architecture, Component Selection & Engineering Requirements Table

| Functional Sub-Block | Circuit Ref | Selected Component / IC | Key Specifications & Ratings | Functional Role & Performance Target |
| --- | --- | --- | --- | --- |
| HVDC Input Interface | +HVDC | Stage 2 Bulk DC Bus Link | +120 V to +375 V DC operating span | Low-impedance high-voltage input rail feeding transformer primary winding and snubber network. |
| Uniform-Freq PWM Controller | U1 | Texas Instruments UC3843 / UCC28C43 (SOIC-8 / DIP-8) | Uniform 65 kHz clock, RT/CT oscillator, cycle-by-cycle CS, direct totem-pole gate out | Generates constant-frequency PWM switching pulses at a uniform 65 kHz rate, providing predictable EMI harmonics and deterministic current-mode control. |
| Oscillator Timing Network | R_T C_T | Precision Thin-Film Resistor + C0G/NP0 Ceramic Capacitor | R_T = 10.0 kΩ (1%), C_T = 2.2 nF (50V C0G) | Sets constant oscillator clock frequency: f_sw approx 1.72 / (R_T x C_T) = 65 kHz uniform clock without frequency jitter. |
| Primary Power Switch | Q1 | STMicroelectronics STP10NK80Z / Infineon IPB80R290P7 | 800 V, 17 A, R_DS(on) = 0.29 Ω, V_DS,peak = 599 V | Main switching element modulated by 65 kHz PWM gate drive. Voltage stress derated to 74.9% of 800V rating (< 75% limit). |
| Primary Current Sensing | R_CS | Current Sense Shunt Resistor Network | Low-inductance metal strip, 0.25 Ω, 1%, 2 W | Monitors primary switch peak current (I_pk = 2.96 A) on UC3843 ISENSE pin (Pin 3) with 1.0 V internal comparator threshold for cycle-by-cycle OCP. |
| RCD Snubber Clamp | D_snub R_snub C_snub | Diodes Inc. US1M + 3x 68 kΩ in parallel (22.6 kΩ, 3W) + 1.0 nF / 1 kV Ceramic | Diode: 1000 V, 1 A, t_rr = 75 ns; Cap: 1 nF / 1000 V ceramic; Clamp: ~216 V clamp level | Damps high-voltage inductive kick caused by transformer leakage inductance (L_lk approx 2.2 uH); clamps peak drain voltage spike safely below 650 V peak. |
| Flyback Power Transformer | TX1 | Ferroxcube PQ26/20 (3C95 Ferrite) / EE25 Core | L_p = 220 uH +/- 10%, B_max = 0.148 T (B_sat = 0.38 T), N_p = 36 T | Stores energy during switch on-time and delivers 72 W continuous power to secondary during off-time (Np:Ns = 6:1 turns ratio). |
| Auxiliary Bias Winding | TX1_AUX | N_aux = 5 T (0.25 mm wire) + Fast Diode (UF4004) & 47 µF Cap | Output: +14.0 V DC bias rail | Sustains UC3843 VCC pin (Pin 7) above UVLO threshold (8.4 V start / 7.6 V min) after initial high-voltage bleed startup. |
| Galvanic Safety Isolation Barrier | TX1_ISO | Furukawa TEX-E Triple-Insulated Wire (TIW) | N_s = 6 T (N_p:N_s = 6:1), Reinforced insulation, >= 6.4 mm creepage | Complies with IEC 62368-1 standard. Provides certified >= 3000 V RMS dielectric withstand between primary hot domain and secondary touch-safe SELV domain. |
| Secondary Schottky Barrier Rectifier | D_SEC | ON Semi / Vishay MBR10100 / MBR20100CT | 100 V, 10 A Schottky Barrier Diode, V_F = 0.60 V, TO-220 with heatsink | Performs high-speed passive rectification of secondary flyback pulses. Highly reliable and robust passive design, eliminating active gate driver IC complexity. |
| Secondary LC Output Filter | C_sec L_sec | 470 µF / 35 V Low-ESR Electrolytic + 2.2 µH Inductor + 22 µF MLCC | Low-ESR filtering, Delta V_ripple < 50 mV pk-pk | Attenuates 65 kHz fundamental switching ripple and high-frequency commutation spikes to produce a quiet intermediate DC rail. |
| Isolated Closed-Loop Feedback | OPTO1 U3 | Everlight PC817 / EL817 Optocoupler + TI TL431 Shunt Regulator | 5000 V RMS optical isolation, 2.495 V internal bandgap, 0.5% | Precision resistor divider samples +24.0 V rail, driving optocoupler LED to pull down UC3843 COMP pin (Pin 1), regulating rail to +/- 1.0%. |
| Intermediate Rail Output | +24V_BUS | Intermediate DC Power Distribution Bus | +24.0 V DC +/- 1.0%, 72.0 W continuous (3.0 A), 84 W peak | Delivers stable intermediate regulated DC power to Stage 4 (Synchronous Buck-Boost Post-Regulator) and secondary auxiliary housekeeping LDOs. |

Note: This table is pre-formatted with standard cell margins, repeating headers, and fixed column widths. You can select the entire table in Word, copy it (Ctrl+C), and paste it (Ctrl+V) directly into your PRD document.