Product Requirement Document (PRD)

| Project Code | VVDI_EVSE |
| --- | --- |
| Project Name | VVDN INDIA , ELECTRIC VEHICLE SUPPLY EQUIPMENT |
| Revision | A1_001 |
| Date | 7th Oct 2026 |

Revision History

| Date | Revision No. | Description | Prepared/Revised By | Reviewed By | Approved By |
| --- | --- | --- | --- | --- | --- |
| 06-Oct-2026 | A0-01 | INITIAL RELEASE | SHIKHA SINGH | ANKIT DUBEY | ANKIT DUBEY |

Customer Sign Off

| CUSTOMER SIGN-OFF FORM |  |  |
| --- | --- | --- |
| Customer | Technical In-Charge | Program In-Charge |
| Name: |  |  |
| Designation: |  |  |
| Email ID: |  |  |
| Address: |  |  |
| Comments: |  |  |
| Signature & Date: |  |  |
| VVDN | Technical In-Charge | Program In-Charge |
| Name: | SHIKHA SINGH | ANKIT DUBEY |
| Designation: | TRAINEE (HARDWARE) | SENIOR HARDWARE ENGINEER |
| Email ID: | shikha.singh@vvdntech.in | ankit.dubey@vvdntech.in |
| Address: |  |  |
| Comments: |  |  |
| Signature & Date: | Shikha singh & 10-04-2026 |  |

Internal Sign-Off

| INTERNAL SIGN-OFF |  |
| --- | --- |
| Level 1 | BU SME |
| Name: | shikha |
| Comments: |  |
| Signature & Date: |  |

Table of Contents

1. Introduction	7

1.1. Purpose	7

1.2. Document Conventions	7

1.3. Intended Audience and Reading Suggestions	7

1.4. Abbreviation	9

1.5. References	10

2. Product Overview	10

2.1. Product - Business Perspective	10

2.2. Product – Technical Perspective	10

Figure 1: Boundary Diagram	11

2.3. Use Cases	14

2.4. Product Interfaces	14

2.4.1. Hardware Interfaces	14

2.4.2. Software Interfaces	16

3. Systems & Domains Involved	18

4. Product Requirement	18

4.1. System Requirements	18

4.2. Mechanical ID Requirement	18

5. VVDN_EVSE SOW and dependencies	18

6. Standard Deliverables & Release Order	19

7. Product Development Plan	20

Phase 6 — Documentation & Handover	20

8. EVSE Architecture	20

Figure 3: Hardware Architecture	21

9. Out of Scope	21

10. Assumptions	22

List of Tables

Table 1  Abbreviation	9

Table 2: Reference	10

Table 3 High Level Technical Requirement	13

Table 4 Product Usecase	14

Table 5 Hardware Interface	16

Table 6  Software Interfaces	18

Table 7: Domain	18

List of Figures

Figure 1: Boundary Diagram	11

Figure 2: Hardware Architecture	21

# Introduction

## Purpose

This document describes the system-level requirements for the design and development of “VVDI_EVSE” for internal VVDN . The document will be used as a reference for further documents and product design. The device's functional and technical requirements have to meet the parameters mentioned in the document. These requirements have been derived from the specification provided by Internal and subsequent discussion.

This document captures all Electrical, Mechanical & Software requirements to be delivered as per the “VVDI_EVSE Charger Requirements - low voltage  Board Only.

VVDI_EVSE  is the project id for this project and it will be used in every document related to this project.

## Document Conventions

This PRD shall be taken as a reference document for any requirements. This document follows Times New Roman Format in size 12 font. The bold-faced text has been used to emphasize section and subsection headings. Highlighting is to point out words in the glossary, and italicized text is used to label and recognize diagrams.

## Intended Audience and Reading Suggestions

This PRD is made for the reference of

- A product manager at VVDI/VVDN to confirm the requirements before the development.

- The engineering team at VVDN for architecture, design, and development for VVDI/VVDN

- System integration and verification team atVVDI/VVDN

- Testing/QA team at VVDN to derive the test cases to satisfy the product functional and certification requirements.

- Production Team at VVDN to define and validate the device test cases from a production perspective.

The suggested sequence for reading the document and proceeding through the sections that are most pertinent to each reader type is described below. Concerned team/members are advised to read the document as per the stated flow:

- Client Team

    - Complete Document

- Architect Team

    - Use Cases, Product Requirements

- Software Design team

    - Use cases, SW Interfaces, Product Requirement

- Software Development Team

    - HW Interfaces, Use cases, SW Interfaces, Protocol Compliance

- Quality Assurance Team

    - Complete Document

- Documentation team

    - Complete document.

- Deployment Team

    - Deployment and Installation Guidelines

## Abbreviation

| Acronym | Definition |
| --- | --- |
| VVDI_EVSE | VVD india, Electric Vehicle Supply Equipment |
| PRD | Product Requirement Document |
| AC | Alternating Current |
| DC | Direct Current |
| QA | Quality Assurance |
| SDD | Software design Document |
| MDD | Mechanical Design Document |
| HDD | Hardware design Document |
| OCPP | Open Charge Point Protocol |
| SRN | Software Release Note. |
| ID | Industrial Design. |
| PT | Potential Transformer |
| CT | Current Transformer |
| RCD | Residual Current Detection |
| GPIO | General Purpose Input Output |
| ADC | Analog to Digital Converter |

#### Table 1  Abbreviation

## References

| SL No | Description (with Version & External Link) |
| --- | --- |
| REF_ID-001 | Title: Description: Author: VVDN/XXXX Signed Date: 28 Sep 2025 Click HereScope of Work Document |

#### Table 2: Reference

# Product Overview

## Product - Business Perspective

An Electric Vehicle Supply Equipment (EVSE) unit serves as a critical interface for safely delivering AC power from the electrical grid to an electric vehicle, adhering strictly to global EV standards. Designed around a robust architecture, the control system integrates an auxiliary power stage, energy metering unit, AC residual current detector (RCD), relay drivers, isolated line voltage sensing across relays, user interfaces, and bi-directional communication modules. Together, these subsystems enable real-time operational monitoring, precise consumption tracking, and secure, reliable power distribution.

At the core of the system is the Control Pilot interface, a primary control conductor that manages essential safety protocols and vehicle-equipment interaction. Connected through vehicle control circuitry to equipment ground, it verifies physical vehicle attachment, regulates power energization and de-energization, advertises available current capacity within safe rating limits, and continuously monitors protective earth (PE) ground integrity. Combined with built-in AC residual current detection and isolated sensing, the system effectively mitigates electrical shock and fire hazards during the charging process.

## Product – Technical Perspective

The EVSE is an IoT-based AC charging system in which the LV controller board works as the main control and monitoring unit. The board takes a regulated 12 VDC input and provides the required 5 VDC and 3.3 VDC supplies for different electronic circuits. It interfaces with important EVSE functions such as Control Pilot (CP), Proximity Pilot (PP), contactor/Relay Drive control and feedback, voltage and current monitoring, temperature sensing, AC/DC leakage detection, self-test, fault monitoring, RFID, Wi-Fi, and status indication through LEDs. The MCU acts as the brain of the board by processing these signals, managing the EVSE charging state, monitoring safety conditions, detecting faults, and providing control signals such as contactor commands and CP/PWM control. The communication interfaces allow the EVSE to share charging status, diagnostic information, and other required data with external systems or the cloud. In simple terms, the LV controller board provides the intelligence of the EVSE and manages its control, monitoring, safety, and communication functions, while the high-voltage AC power switching and protection section is outside the scope of the LV controller board.

The primary EVSE functionality includes:

• Regulated electrical current: Ensures that the optimal current is provided and falls within the maximum current the EV can handle.

• AC residual current detection (RCD)

• Relay drive and weld detection

• Energy metering

• Automatic disconnect: When a critical hardware fault is detected, the LV controller immediately initiates a power shutdown to ensure safe operation. The controller continuously monitors conditions such as leakage, over-temperature, abnormal CP/PP signals, and contactor faults. If a critical fault occurs, the controller disables the Relay drive, causing the external relay to open and disconnect the AC supply from the EV. The controller also checks the relay feedback to confirm that the power has been successfully disconnected. This helps prevent potential hazards such as battery damage, electrical short circuits, overheating, or fire.

### Figure 1: Boundary Diagram

AC leakage, residual current detection (RCD):

The primary requirement in protecting during EV charging is the ability to detect AC residual currents and thereby mitigate the risk of electric shock or fire.

Low standby EVSE:

The EVSE shall consume a maximum of 5 W of power during standby/idle conditions when the EV is not charging.

Efficient relay drive control :

The EVSE uses an efficient relay-drive architecture to control inductive relay coils with minimal power consumption. Since relays may remain energized for long durations, optimized coil driving helps reduce power loss, heat generation, and thermal stress. wered, an efficient drive solution is preferred to avoid thermal problems.

Contact weld detection:

The EVSE monitors relay output voltage to detect welded/fused contacts or unintended voltage at the plug, ensuring safe operation. This check should be performed every time the relay is opened to confirm that power is actually disconnected.

Self test:

It verifies the health and integrity of critical EVSE hardware and interfaces, including sensing, protection, relay/contactor control, memory, watchdog, and communication circuits. It ensures the controller is in a safe and reliable operating condition before charging is enabled.

Control Pilot Interface :

The CP interface provides EV connection detection, charging-state determination, PWM-based current control, and EVSE–EV communication through the Control Pilot signal.

RFID  Interface :

The RFID interface is used to identify and authenticate the user by reading an RFID card or tag. Once the user is verified, it sends the information to the main controller to allow or deny charging.

LED Driver Interface :

The LED driver interface controls the EVSE LEDs to show power, charging, standby, and fault status. The main controller sends control signals to the LED driver, which then operates the required LEDs accordingly.

WIFI Interface :

The Wi-Fi interface provides wireless communication between the EVSE controller and the cloud/server for data exchange. It supports remote monitoring, status updates, and control commands.

| S.No | Parameters | Inputs For LV Board Design |
| --- | --- | --- |
| 1 | PCBA Operating Temperature | –30°C to 55°C |
| 2 | Continuous Current Rating | 16 Amps |
| 3 | Input Voltage | 180–285V AC, 50 Hz |
| 4 | Charging Protocol | SAE J1772 (Control Pilot Protocol) |
| 5 | Connector Interface | Type-2 EV charging connector; CP, PP and PE signal interfaces |
| 6 | RFID Control | RFID-based user authentication / charging enable-disable |
| 7 | PCBA Environmental Rating | Operating Temperature: -30°C to 85°C Altitude Operation Without De-Rating : Up to 3,000m. Humidity: 0-100% RH non-condensing |
| 8 | Safety Features | 1) Under and over-voltage protection 2) Under and over temp protection 3) Ground fault circuit interruption 4) AC and DC leakage Protection 5) Overcurrent protection 6) Relay weld and open detection 7) Over Temperature Protection 8) Emergency Switch Control |
| 9 | LED Drive Control | MCU-controlled LED driver interface for charger status indication: Power/Standby, Charging, Fault and other system states |
| 10 | Wifi | Wi-Fi interface for communication between LV EVSE controller and cloud/backend system; charging status, diagnostics and control commands |

#### Table 3 High Level Technical Requirement

## Use Cases

The primary EVSE functionality includes

| S.N. | Usecase |
| --- | --- |
| 1 | Control Pilot State detection |
| 2 | Voltage and current monitoring through I2C |
| 3 | AC/DC leakage detection |
| 4 | Fault detection and protection |
| 5 | RFID-based user authentication |
| 6 | LED status indication |
| 7 | PP monitoring |
| 8 | Relay/contactor drive control |
| 9 | Temperature monitoring |
| 10 | Self-test |
| 11 | Wi-Fi communication |
| 12 | System reset and fault recovery |

#### Table 4 Product Usecase

## Product Interfaces

## Hardware Interfaces

AC leakage, residual current detection (RCD):

The EVSE LV board checks whether any unwanted AC current is leaking from the normal current path. If the leakage becomes higher than the safe limit, the controller detects the fault, stops charging, and disconnects the relay/contactor to keep the user and EV safe.

Low standby EVSE:

When the EV is not charging, the LV controller keeps the EVSE in standby mode and reduces power consumption by putting non-essential circuits into low-power states. Essential functions such as monitoring, communication, and fault detection remain active so the EVSE can quickly respond when charging is requested.

Efficient relay drive control :

The LV controller drives the relay/contactor using an efficient driver circuit that provides the required coil current while minimizing power consumption. During long ON periods, the drive reduces unnecessary coil power and heat generation, improving relay reliability and overall EVSE efficiency.

Contact weld detection:

The LV controller checks the relay/contactor output voltage after the contactor is switched OFF. If voltage is still detected, it indicates that the contacts may be welded or stuck, so the controller generates a fault and prevents further charging for safety.

Self test:

The controller first gives a small test current or voltage to the sensing/protection circuit and checks the response it receives back. If the feedback is within the expected range, the circuit is considered OK; if not, the controller detects a fault and does not allow charging.

Control Pilot Interface :

The CP Connector on the LV Board serves as the physical and electrical interface that connects the control board's Control Pilot (CP) circuitry directly to the vehicle through the  EV GUN. It handles the bi-directional transmission of the PWM (Pulse Width Modulation) pilot signal required by EVSE standards. Through this connector, the STM32 microcontroller monitors voltage state transitions (e.g., State A to State F) via the CP Circuit to detect vehicle presence, request state  while simultaneously communicating the maximum allowable charging current back to the Electric Vehicle

RFID  Interface :

RFID interface in an EVSE is used for user authentication before starting a charging session.The RFID reader communicates with the MCU through SPI, UART, or I²C, depending on the reader module.The MCU verifies the RFID card/tag and, if authorized, enables the charging process through the contactor /relay control.

LED Driver Interface :

The LED driver interface in an EVSE controls the status LEDs used to indicate conditions such as power, charging, fault, and communication status.The MCU sends control signals to the LED driver, which provides the required current to the LEDs.It allows the EVSE to show different colors or blinking patterns according to the charging/state-machine status.

WIFI Interface

The Wi-Fi interface provides wireless communication between the EVSE controller and a cloud/mobile application for remote monitoring and control.The MCU communicates with the Wi-Fi module through UART, SPI, or another supported interface.It enables functions such as status monitoring, charging data transmission, diagnostics, and firmware updates.

| Sl. No | Entity1 | Entity2 | Interface | Usage Description |
| --- | --- | --- | --- | --- |
|  | LV board | Main controller | Voltage and current monitoring | Voltage/current measurement, charging parameter monitoring, and fault detection . |
|  | LV board | Main controller | Relay Drive Control | Relay/contactor switching, load control, and safe power disconnection. |
|  | LV board | Main controller | AC and DC Leakage and Self Test | Leakage fault detection, protection verification, and system health monitoring. |
|  | LV board | Main controller | Control Pilot Interface | Charging state determination ev connection detection, and charging control. |
|  | LV board | Main controller | RFID Interface | User authentication,access control ,charging authorization |
| 6 | LV board | Main controller | Wifi Module | Wireless connectivity, cloud communication, and remote monitoring/control. |
| 7 | LV board | Main controller | LED Drive Control | State indication, charging-state indication, and fault indication |
| 8 | LV board | Main controller | Temperature Monitoring | Temperature sensing, thermal protection & over-temperature fault detection. |
| 9 | USER | AGD Control Card | Reset | Controller reset, fault recovery, and system reinitialization. |
| 10 | USER | AGD Control Card | LED Drive Control | LED status indication, system status indication, and fault indication. |

#### Table 5 Hardware Interface

## Software Interfaces

EVSE contains real-time Control and Monitoring Unit (STM32) and Cloud Communication Unit (ESP32) having a Wi-Fi interface for cloud communication and BLE for mobile app communication. STM32 and ESP32 talk to each other serially over UART.

The following table describes the interface, protocol/mode, and usage of each interface.

| S. No | Entity1 | Entity2 | Interface | Interface Protocol/Mode | Usage Description |
| --- | --- | --- | --- | --- | --- |
| 1. | LV board | Main controller | Voltage and current monitoring | Analog / ADC /ADC-capable GPIO | Voltage/current measurement, charging parameter monitoring, and fault detection. |
| 2. | LV board | Main controller | Relay Drive Control | GPIO +TIMER | Relay/contactor switching, load control, and safe power disconnection. |
| 3. | LV board | Main controller | AC and DC Leakage and Self Test | GPIO | Leakage fault detection, protection verification, and system health monitoring. |
| 4. | LV board | Main controller | Control Pilot Interface | PWM OUTPUT + ADC INPUT | Charging state determination ev connection detection, and charging control. |
| 5. | LV board | Main controller | RFID Interface | SPI / I²C | User authentication,access control ,charging authorization |
| 6. | LV board | Main controller | Wifi Module | UART | Wireless connectivity, cloud communication, and remote monitoring/control. |
| 7 | LV board | Main controller | Temperature Monitoring | ADC INPUT | Temperature sensing, thermal protection, and over-temperature fault detection. |
| 8. | USER | AGD Control Card | Reset | GPIO / Digital Input | Controller reset, fault recovery, and system reinitialization. |
| 9 | USER | AGD Control Card | LED Drive Control | GPIO / PWM | LED status indication, system status indication, and fault indication. |

#### Table 6  Software Interfaces

# Systems & Domains Involved

The below domains are involved in this project.

| Domain Names | System 1 (EVSE Device) |
| --- | --- |
| Hardware - HW | Yes |
| Mechanical - ME | Yes |
| Firmware | No |
| SystemApp - SAP | No |
| MobileApp - MAP | No |
| CloudApp - CAP | No |
| PC App - PAP | No |
| (if any) | Test Firmware for all interface validation |

#### Table 7: Domain

# Product Requirement

## System Requirements

click here

## Mechanical ID Requirement

NOT in scope

# VVDN_EVSE SOW and dependencies

The following dependencies are included in the design and development of the VVDI_EVSE charger.

- Existing HV EVSE board interface specifications for control, feedback, fault, and I²C signals will be required during schematic and interface design

- HV board voltage and current measurement data format, I²C address, register map, and communication protocol will be required for MCU integration.

- Relay/contactor drive interface and weld-feedback specifications from the existing HV board will be required during LV interface design.

- RCD will require two signals The RCD self-test signal will be provided as an input to the MCU GPIO, while the AC and DC leakage detection signals will be provided as RCD outputs. These outputs will be LOW by default and transitioned to high during AC or DC leakage events.

- For the ground fault signal, the Earth Detection Fault signal will be provided as an input to the MCU GPIO. The MCU will receive a PWM signal when the earth is connected. When the earth is disconnected, the signal will transition to HIGH.

- ESP32 UART interface requirements will be defined for Wi-Fi/IoT communication during hardware and firmware integration.

- RFID controller and SPI interface requirements will be finalized before schematic design and firmware integration.

- EEPROM and NOR Flash memory requirements, including data storage structure for configuration, calibration, event, and fault data, will be defined during firmware development.

- EVSE state-machine behavior, fault-handling logic, and safe-state conditions will be defined during firmware development.

- One NTC based temperature monitoring input will come to MCU which should be connected the required MCU pin .

# Standard Deliverables & Release Order

| System Name | Domain Name | Deliverables |
| --- | --- | --- |
| VVDI_EVSE |  | - PRD |
| EVSE Charger | Hardware | - HDD - Schematics - Bill of Materials - PCB Layout - Gerber [Fab & Assembly package] - Test Report |

# Product Development Plan

Phase 1 — Requirement & Architecture and Class A BOM

Definition of functional and non-functional requirements; system architecture design; block diagram and interface specification.

Phase 2 — Component Selection & Circuit Design

Component research and selection; power budget and thermal analysis; circuit design and simulation; design reviews.

Phase 3 — Schematic Design and  Review

Complete schematic design; cross-check against requirements; peer review; design-rule checks.

Phase 4 — PCB Layout

PCB stack-up and layer definition; component placement; routing with signal integrity and EMI consideration; DFM review.

Phase 5 — Design and Validation Plan

PCB fabrication and assembly; initial power-on and functionality verification; debugging and rework as needed.

# EVSE Architecture

The following section shows the Hardware architecture involved in the development of Type 1 Level 2 EV chargers for the INDIAN market.

NOTE: The more detailed hardware architecture will be captured in the hardware design document with selected components and their connections.

-

### Figure 2: Hardware Architecture

# Out of Scope

The following points are not included in the scope of the project:

- HV Board and HV Power section

- HV relay/contactor circuitry

- HV voltage/current sensing circuits

# Assumptions

The following points are considered as assumptions for further project execution and may have dependencies :

- Existing HV EVSE board interface specifications for control, feedback, fault, and I²C signals will be required during schematic and interface design

- HV board voltage and current measurement data format, I²C address, register map, and communication protocol will be required for MCU integration.

- Relay/contactor drive interface and weld-feedback specifications from the existing HV board will be required during LV interface design.

- RCD will require two signals The RCD self-test signal will be provided as an input to the MCU GPIO, while the AC and DC leakage detection signals will be provided as RCD outputs. These outputs will be LOW by default and transitioned to high during AC or DC leakage events.

- For the ground fault signal, the Earth Detection Fault signal will be provided as an input to the MCU GPIO. The MCU will receive a PWM signal when the earth is connected. When the earth is disconnected, the signal will transition to HIGH.